UPDATE of TABLE 7-6 from GARD – comparison of AMD process criteria

| Selection criteria | Chemical precipitation | Membrane treatment | Ion exchange – KNeW process | Biological sulfate removal |
| --- | --- | --- | --- | --- |
| Proven technology on commercial scale | Widely proven with large plants | ell proven with some large plants | Only at pilot plant stage | Proven with a limited number of plants |
| Water recovery | >95% | >95% | >90% | >95% |
| Waste sludge / brine production | Large waste sludge production | Large sludge and brine production | Minimal waste production | Small waste sludge production |
| By-products recovery | Potential for CaSO₄ recovery | CaSO₄ recovered in some areas | Fertilizer recovery | Potential for S recovery |
| Chemical dosing | High chemicals dosing | Limited chemicals dosing | High chemicals dosing | Carbon source dosing |
| Energy usage | Moderate energy usage | High energy usage | Low energy usage | Moderate energy usage (heating) |
| Sodium and chloride removal | Cannot remove | Removes to sludge and long term storage | Removes to product | Cannot remove |
| Reliable and robust performance | Robust process | Process good performance, but sensitive to pre-treatment | Pilot plant work proceeding to establish parameters | Biological process sensitive to toxics, fluctuating feed water quality and environmental conditions |
| Capex - $/mᶟ/day | $ 300 – 1,250 | $ 500 – 1,000 | $ 400 - 750 | $ 800 – 1,500 |
| Opex - $/mᶟ/day | Neg $ 0.2 – 1.5 | Neg $ 0.5 – 1.0 | Pos $ 1.0 | Neg $ 0.7 – 1.5 |